[image: image1.wmf]CHEMSTRESS CONSULTANT COMPANY


NORTHERN OPERATIONS
MIDWESTERN OPERATIONS
SOUTHERN OPERATIONS
EASTERN OPERATIONS
September 9, 2016
Board of Directors

Silver Lake Estates
Attn:  Mr. Richard G. Lubinski

2915 Silver Lake Blvd.
Silver Lake, Ohio 44224
Subject:
Silver Lake Estates


Site Rehabilitation Project

North Timber and Boathouse Concrete Wall Inspections

As requested, Chemstress Consultant Company performed a visual and physical inspection of above subject retaining walls.  The following will summarize the findings of the structural issues and evaluate the useful life of the walls.

The two walls were visited approximately seven (7) years ago.  The two walls varied in height and construction types.  
1. The North retaining wall is a timber wall and is estimated to be approximately 30 years or more in age and has suffered from normal exposure to the elements, excessive water exposure, and insects.  Walls of this construction type typically are expected to have a life expectancy of approximately 15 years.  The condition of the North wall would classify as poor, and deemed to be at risk of isolated collapse and has no considerable useful life.

2. The Boathouse retaining wall is a concrete shoreline wall with heights generally in the range of only two to three feet in height.  This wall is estimated to be in the range of a minimum 40 years or more in age and has suffered differential movement, spalled concrete, and cracking.  Walls of this construction type typically are expected to have a life expectancy of approximately 30 years.  The condition of the Boathouse retaining wall would classify as poor, and has technically failed in locations where the wall had shifted forward and is no longer functioning as designed.  Though this wall could continue to slowly worsen, it would not classify as having any useful life due to the areas of failure.


If additional information is required, please feel free to contact me at (330) 715-6226.

Sincerely,

[image: image2.jpg]



Bradley W. Lightfoot, P.E.

Project Engineer

39 S. MAIN STREET


AKRON, OH 44308


(330) 535-5591 FAX (330) 434-3400








Page 1 of 1
Page 2 of 2

